And UHF development documentation

catalogue

And UHF development documentation	1

Document description	1

Demo, and the running environment	1

Description of the UHF call process	1

UHF initialise	1

UHF take inventory	2

Single label storage	2

Multi-label disk storage	2

The UHF power setting	3

Read and write power Settings	3

Read and write power acquisition	3

The UHF tag is read out	3

The UHF label writes to the label	3

# Document description

This document applies to the android studio development environment

Demo, operating environment:
android studio Version: 2021.2.1
The jdk version of 1.8.0_333

Preparation: introduce ModuleApi in demo under your project, this file is a library interface of module package, used to call device function, introduce so file in jniLibs under main directory, this file is also included in demo, this file is module driven and must be added.

# Description of the UHF call process

The equipment call cycle follows the initiation of reading, writing, configuration and other operations. When it is not in use, it needs to be disconnected. If the connection is not disconnected, the module will cause additional power loss caused by standby.

# UHF initialise

Before each use, the device needs to connect to the module, and only needs to call once each time before the exit program is disconnect. The connection code refers to the OpenTask asynchronous thread in MainActivity in the demo. It is recommended to put this action in the sub-thread processing, and all function calls can continue based on this initialization success.

UHFReader.getInstance ().connect(MainActivity.this );

When quitting the program or not using UHF, call:

UHFReader.getInstance ().disConnect();

# UHF take inventory

The inventory function is mostly used for the need to inventory multi-label, inventory points multi-label and single label inventory function

# Single label storage

instance:

UHFReaderResult<UHFTagEntity> readerResult = UHFReader.getInstance ().singleTagInventory();

readerResult.getData(); / / labeled data

Immediately after the inventory is called, the label data returns only one label at a time and only EPC content

Multi-label disk storage

Multi-label disk storage needs to first set the callback to receive the data, and then open the inventory

Example of a data callback:

UHFReader.getInstance ().setOnInventoryDataListener(new OnInventoryDataListener() {
@Override
public void onInventoryData (List<UHFTagEntity> tagEntityList) {
/ / tagEntityList is a data callback class from which tag data
}
});

Open the inventory example:

UHFReaderResult<Boolean> readerResult = UHFReader.getInstance ().startInventory();

The UHF power setting

Read and write power Settings

UHFReaderResult<Boolean> result = UHFReader.getInstance ().setPower(30);

Read and write power acquisition

UHFReaderResult<Integer> result = UHFReader.getInstance ().getPower();

The UHF tag is read out

Read required parameters: access password + read area + read start address + read length

instance:

UHFReader.getInstance (). Read (password, membank, address, wordCount, specify label read, if specified incoming filtered entity class);

Note that when reading the EPC area, you need to start from address 2. Address 0 and 1 are protocol-based and negligible nonreading

Address 0 is the inventory CRC check, address 1 is the inventory length control: determines the EPC length of the multi-label disk

The UHF label writes to the label

Write required parameter: access password + write area + write start address + read length

UHFReader.getInstance (). Write (password, membank, address, wordCount, hexData, whether to specify the label write, if specified, the incoming entity class);

Note: Address 1 is the memory length control: determines the length of the multi-label memory EPC